SIMATS ENGINEERING

CO3- AT2 - Design Task - Medium

COURSE NAME: Advance Wireless Communication Systems / 5G / 6G Communication Systems

COURSE CODE: ECA56

COURSE OUTCOME COVERED

CO: 3 - Design spectrum access strategies, waveform generation methods, and  multiple access techniques for efficient wireless transmission systems. (BL6)

Assessment Tool Number : CO3_AT2

Assessment Tool Weightage : 35%

Assessment Tool : Design Task

Duration : 180 Minutes

Marks : 100 Marks

Type of Assessment Conduction : At Home

Evaluation Type : GitHub

Result Generation : Students design and justify appropriate wireless  communication solutions by applying engineering

principles to meet specified requirements.

Topic Covered :

Spectrum management, licensed/unlicensed spectrum, dynamic spectrum access, cognitive radio, OFDMA, resource block allocation, GFDMA, NOMA, power- domain  multiplexing, SIC, and IoT applications. Advanced waveforms and wireless  technologies including GFDM, FBMC, UFMC, mmWave propagation, path loss and  penetration, millimeter-wave antenna systems, MIMO, and Massive MIMO  fundamentals.

Short description about assessment: Students design a system / model based on  given requirements to assess creativity and application skills. This question paper  consists of a total of 40 questions, out of which each student is required to answer any  2 questions. Each question carries 50 marks, and the paper carries a total of 100  marks

QUESTIONS: 2 (2 X 50 = 100) Total Marks: 100 Duration: 3hrs

5. Design an edge AI workflow for intelligent wireless data filtering. (BL6)

27. Design a low-power communication model for battery-operated IoT devices. (BL6)

Code of Conduct:

I MOHITH M 192412444(Name / Reg No) certify that this submission is my original  work and that I have adhered to the guidelines specified for this assessment. I understand that any violation of academic integrity rules will result in disciplinary  action.

Signature of the student:

RUBRICS FOR CALCULATION:

| Design Task |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Criteria | Weightag e | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| Understanding of Design | 20% | Demonstrates deep understanding of design objectives, constraints, and context | Shows good understanding with minor gaps | Partial understanding; misses key aspects | Lacks understandin g of the design context |
| Evaluation of Design | 25% | Provides in depth critique identifying strengths, weaknesses, and design gaps | Identifies key issues with reasonable analysis | Limited critique; mostly descriptive feedback | No meaningful critique provided |
| Justification | 20% | Supports critique with strong reasoning, evidence, and relevant principles | Provides justification with some supporting evidence | Weak or unclear justification | No justification for feedback |
| Suggestions for Improvement | 20% | Offers innovative, practical, and well-justified improvement suggestions | Provides useful suggestions with minor gaps | Suggestions are limited or partially relevant | No constructive suggestions provided |
| Communicatio n | 15% | Communicates feedback clearly, professionally, and engages actively in discussion | Communicates well with minor issues | Communication lacks clarity or engagement | Poor communicatio n and minimal participation |

5.  Design an edge AI workflow for intelligent wireless data filtering.

Aim:

To design and simulate an Edge AI workflow for intelligent wireless data filtering and analyze the effectiveness of filtering noisy wireless sensor data before transmission.

Program Code:

% Edge AI Workflow for Intelligent Wireless Data Filtering

clc;

clear;

close all;

% Generate wireless sensor data

t = 0:0.01:10;

signal = sin(2*pi*0.5*t);

% Add random noise

noisySignal = signal + 0.5*randn(size(signal));

% Edge AI filtering using moving average filter

windowSize = 10;

filteredSignal = movmean(noisySignal, windowSize);

% Plot signals

figure;

plot(t, noisySignal, 'r');

hold on;

plot(t, filteredSignal, 'b', 'LineWidth', 1.5);

plot(t, signal, 'g--', 'LineWidth', 1);

xlabel('Time (s)');

ylabel('Amplitude');

title('Edge AI Workflow for Intelligent Wireless Data Filtering');

legend('Noisy Wireless Data', 'Filtered Data', 'Original Signal');

grid on;

% Calculate Mean Squared Error

mseBefore = mean((signal - noisySignal).^2);

mseAfter = mean((signal - filteredSignal).^2);

disp(['MSE Before Filtering = ', num2str(mseBefore)]);

disp(['MSE After Filtering = ', num2str(mseAfter)]);

### Result:

The Edge AI workflow for intelligent wireless data filtering was successfully simulated in MATLAB. The moving average filter reduced noise from the wireless sensor signal, producing a filtered output that closely matched the original signal. The Mean Squared Error (MSE) after filtering was lower than before filtering, demonstrating that edge-based intelligent filtering improves data quality and reduces unnecessary wireless data transmission.

### 27. Design a low-power communication model for battery-operated IoT devices.

### Aim:

To design and simulate a low-power communication model for battery-operated IoT devices and evaluate the reduction in energy consumption achieved through duty-cycled communication.

Program Code:

% Low-Power Communication Model for Battery-Operated IoT Devices

clc;

clear;

close all;

% Time duration (seconds)

t = 1:100;

% Power consumption (mW)

activePower = 50;      % Active transmission power

sleepPower = 5;        % Sleep mode power

% Duty cycle (20% active, 80% sleep)

dutyCycle = 0.2;

% Conventional communication (always active)

powerConventional = activePower * ones(size(t));

% Low-power communication model

powerLow = sleepPower * ones(size(t));

activeIndex = mod(t,5)==0;      % Device becomes active periodically

powerLow(activeIndex) = activePower;

% Plot

figure;

plot(t,powerConventional,'r','LineWidth',1.5);

hold on;

plot(t,powerLow,'b','LineWidth',1.5);

xlabel('Time (s)');

ylabel('Power Consumption (mW)');

title('Low-Power Communication Model for Battery-Operated IoT Devices');

legend('Conventional Communication','Low-Power Duty-Cycled Communication');

grid on;

% Average power consumption

avgConventional = mean(powerConventional);

avgLow = mean(powerLow);

disp(['Average Conventional Power = ', num2str(avgConventional), ' mW']);

disp(['Average Low-Power Consumption = ', num2str(avgLow), ' mW']);

Output:

### Result:

The low-power communication model for battery-operated IoT devices was successfully simulated in MATLAB. The duty-cycled communication model showed significantly lower average power consumption than continuous communication. The simulation demonstrates that periodically switching the IoT device between active and sleep modes effectively reduces energy usage and increases battery life, making it suitable for low-power wireless IoT applications.